TRAMA DESCRIPCIÓN DE BASE DE DATOS

Frame    Time         Src MAC Addr    Dst MAC Addr    Protocol    Description

312      280.960000   0009E84D07F2    0009E84D0FA6    OSPF        Message = Database

Src Other Addr     Dst Other Addr     Type Other Addr

172.13.0.2         172.13.0.1

Frame: Base frame properties

Frame: Time of capture = 14/11/02 10:27:40.714

Frame: Time delta from previous physical frame: 2000 microseconds

Frame: Frame number: 312

Frame: Total frame length: 146 bytes

Frame: Capture frame length: 146 bytes

Frame: Frame data: Number of data bytes remaining = 146 (0x0092)

ETHERNET: ETYPE = 0x0800 : Protocol = IP:  DOD Internet Protocol

ETHERNET: Destination address : 0009E84D0FA6

ETHERNET: .......0 = Individual address

ETHERNET: ......0. = Universally administered address

ETHERNET: Source address : 0009E84D07F2

ETHERNET: .......0 = No routing information present

ETHERNET: ......0. = Universally administered address

ETHERNET: Frame Length : 146 (0x0092)

ETHERNET: Ethernet Type : 0x0800 (IP:  DOD Internet Protocol)

ETHERNET: Ethernet Data: Number of data bytes remaining = 132 (0x0084)

IP: ID = 0x3E; Proto = OSPF IGP; Len: 132

IP: Version = 4 (0x4)

IP: Header Length = 20 (0x14)

IP: Precedence = Internetwork Control

IP: Type of Service = Normal Service

IP: Total Length = 132 (0x84)

IP: Identification = 62 (0x3E)

IP: Flags Summary = 0 (0x0)

IP: .......0 = Last fragment in datagram

IP: ......0. = May fragment datagram if necessary

IP: Fragment Offset = 0 (0x0) bytes

IP: Time to Live = 1 (0x1)

IP: Protocol = Open Shortest Path First IGP

IP: Checksum = 0x6006

IP: Source Address = 172.13.0.2

IP: Destination Address = 172.13.0.1

IP: Data: Number of data bytes remaining = 112 (0x0070)

OSPF: Message = Database Description

page 27

Network Monitor trace  Thu 11/14/02 10:41:44  Capture1.txt

OSPF: Version = 2 (0x2)

OSPF: OSPF Packet Type = Database Description

OSPF: Packet Length = 112 (0x70)

OSPF: Source Router ID = 172.33.0.1

OSPF: Area ID = 0.0.0.1

OSPF: Checksum = 0x3F78

OSPF: Authentication Type = None

OSPF: Authentication = 0x0000000000000000

OSPF: Options = 66 (0x42)

OSPF: ......1. = Processes AS External routes

OSPF: .....0.. = Does not support MOSPF

OSPF: ....0... = Does not process Not-So-Stubby-Area advertisements

OSPF: ...0.... = Does not process external attributes LSAs

OSPF: ..0..... = Does not support demand circuit extensions

OSPF: Flag Summary = 2 (0x2)

OSPF: .......0 = Slave

OSPF: ......1. = More database descriptions will follow

OSPF: .....0.. = Not the first Packet of database descriptions

OSPF: Database Sequence Number = 6883 (0x1AE3)

OSPF: Age = 40 (0x28)

OSPF: Options = 34 (0x22)

OSPF: ......1. = Processes AS External routes

OSPF: .....0.. = Does not support MOSPF

OSPF: ....0... = Does not process Not-So-Stubby-Area advertisements

OSPF: ...0.... = Does not process external attributes LSAs

OSPF: ..1..... = Supports demand circuit extensions

OSPF: Link State Advertisement Type = Router Links

OSPF: Router ID = 172.33.0.1

OSPF: Router ID = 172.33.0.1

OSPF: Sequence Number = 2147483649 (0x80000001)

OSPF: Link State Checksum = 0x55AF

OSPF: Length = 48 (0x30)

OSPF: Age = 20 (0x14)

OSPF: Options = 34 (0x22)

OSPF: ......1. = Processes AS External routes

OSPF: .....0.. = Does not support MOSPF

OSPF: ....0... = Does not process Not-So-Stubby-Area advertisements

OSPF: ...0.... = Does not process external attributes LSAs

OSPF: ..1..... = Supports demand circuit extensions

OSPF: Link State Advertisement Type = Summary Link (IP Network)

OSPF: Network Address = 172.3.0.0

OSPF: Router ID = 172.33.0.1

OSPF: Sequence Number = 2147483649 (0x80000001)

OSPF: Link State Checksum = 0x9D1E

OSPF: Length = 28 (0x1C)

OSPF: Age = 30 (0x1E)

OSPF: Options = 34 (0x22)

OSPF: ......1. = Processes AS External routes

OSPF: .....0.. = Does not support MOSPF

OSPF: ....0... = Does not process Not-So-Stubby-Area advertisements

OSPF: ...0.... = Does not process external attributes LSAs

OSPF: ..1..... = Supports demand circuit extensions

OSPF: Link State Advertisement Type = Summary Link (IP Network)

page 28

Network Monitor trace  Thu 11/14/02 10:41:44  Capture1.txt

OSPF: Network Address = 172.33.0.0

OSPF: Router ID = 172.33.0.1

OSPF: Sequence Number = 2147483649 (0x80000001)

OSPF: Link State Checksum = 0x207F

OSPF: Length = 28 (0x1C)

OSPF: Age = 20 (0x14)

OSPF: Options = 34 (0x22)

OSPF: ......1. = Processes AS External routes

OSPF: .....0.. = Does not support MOSPF

OSPF: ....0... = Does not process Not-So-Stubby-Area advertisements

OSPF: ...0.... = Does not process external attributes LSAs

OSPF: ..1..... = Supports demand circuit extensions

OSPF: Link State Advertisement Type = Summary Link (IP Network)

OSPF: Network Address = 172.43.0.0

OSPF: Router ID = 172.33.0.1

OSPF: Sequence Number = 2147483649 (0x80000001)

OSPF: Link State Checksum = 0xB1E2

OSPF: Length = 28 (0x1C)

00000:  00 09 E8 4D 0F A6 00 09 E8 4D 07 F2 08 00 45 C0   ..èM.¦..èM.ò..EÀ

00010:  00 84 00 3E 00 00 01 59 60 06 AC 0D 00 02 AC 0D   .?.>...Y`.¬...¬.

00020:  00 01 02 02 00 70 AC 21 00 01 00 00 00 01 3F 78   .....p¬!......?x

00030:  00 00 00 00 00 00 00 00 00 00 05 DC 42 02 00 00   ...........ÜB...

00040:  1A E3 00 28 22 01 AC 21 00 01 AC 21 00 01 80 00   .ã.(".¬!..¬!..?.

TRAMA PETICIÓN DE ESTADO DE ENLACE

Frame    Time         Src MAC Addr    Dst MAC Addr    Protocol    Description

314      280.962000   0009E84D0FA6    0009E84D07F2    OSPF        Message = Link State

Src Other Addr     Dst Other Addr     Type Other Addr

172.13.0.1         172.13.0.2

Frame: Base frame properties

Frame: Time of capture = 14/11/02 10:27:40.716

Frame: Time delta from previous physical frame: 1000 microseconds

Frame: Frame number: 314

Frame: Total frame length: 106 bytes

Frame: Capture frame length: 106 bytes

Frame: Frame data: Number of data bytes remaining = 106 (0x006A)

ETHERNET: ETYPE = 0x0800 : Protocol = IP:  DOD Internet Protocol

ETHERNET: Destination address : 0009E84D07F2

ETHERNET: .......0 = Individual address

ETHERNET: ......0. = Universally administered address

ETHERNET: Source address : 0009E84D0FA6

ETHERNET: .......0 = No routing information present

page 31

Network Monitor trace  Thu 11/14/02 10:41:44  Capture1.txt

ETHERNET: ......0. = Universally administered address

ETHERNET: Frame Length : 106 (0x006A)

ETHERNET: Ethernet Type : 0x0800 (IP:  DOD Internet Protocol)

ETHERNET: Ethernet Data: Number of data bytes remaining = 92 (0x005C)

IP: ID = 0x3D; Proto = OSPF IGP; Len: 92

IP: Version = 4 (0x4)

IP: Header Length = 20 (0x14)

IP: Precedence = Internetwork Control

IP: Type of Service = Normal Service

IP: Total Length = 92 (0x5C)

IP: Identification = 61 (0x3D)

IP: Flags Summary = 0 (0x0)

IP: .......0 = Last fragment in datagram

IP: ......0. = May fragment datagram if necessary

IP: Fragment Offset = 0 (0x0) bytes

IP: Time to Live = 1 (0x1)

IP: Protocol = Open Shortest Path First IGP

IP: Checksum = 0x602F

IP: Source Address = 172.13.0.1

IP: Destination Address = 172.13.0.2

IP: Data: Number of data bytes remaining = 72 (0x0048)

OSPF: Message = Link State Request

OSPF: Version = 2 (0x2)

OSPF: OSPF Packet Type = Link State Request

OSPF: Packet Length = 72 (0x48)

OSPF: Source Router ID = 172.43.0.2

OSPF: Area ID = 0.0.0.1

OSPF: Checksum = 0xF07D

OSPF: Authentication Type = None

OSPF: Authentication = 0x0000000000000000

OSPF: Link State Type = 0x0000

OSPF: External Network Address = 172.33.0.1

OSPF: Router ID = 172.33.0.1

OSPF: Link State Type = 0x0000

OSPF: External Network Address = 172.43.0.0

OSPF: Router ID = 172.33.0.1

OSPF: Link State Type = 0x0000

OSPF: External Network Address = 172.33.0.0

OSPF: Router ID = 172.33.0.1

OSPF: Link State Type = 0x0000

OSPF: External Network Address = 172.3.0.0

OSPF: Router ID = 172.33.0.1

00000:  00 09 E8 4D 07 F2 00 09 E8 4D 0F A6 08 00 45 C0   ..èM.ò..èM.¦..EÀ

00010:  00 5C 00 3D 00 00 01 59 60 2F AC 0D 00 01 AC 0D   .\.=...Y`/¬...¬.

00020:  00 02 02 03 00 48 AC 2B 00 02 00 00 00 01 F0 7D   .....H¬+......ð}

00030:  00 00 00 00 00 00 00 00 00 00 00 00 00 01 AC 21   ..............¬!

00040:  00 01 AC 21 00 01 00 00 00 03 AC 2B 00 00 AC 21   ..¬!......¬+..¬!

TRAMA ACTUALIZACIÓN  DE ESTADO DE ENLACE

Frame    Time         Src MAC Addr    Dst MAC Addr    Protocol    Description

317      280.964000   0009E84D07F2    0009E84D0FA6    OSPF        Message = Link State

Src Other Addr     Dst Other Addr     Type Other Addr

172.13.0.2         172.13.0.1

Frame: Base frame properties

Frame: Time of capture = 14/11/02 10:27:40.718

Frame: Time delta from previous physical frame: 1000 microseconds

Frame: Frame number: 317

Frame: Total frame length: 194 bytes

Frame: Capture frame length: 194 bytes

Frame: Frame data: Number of data bytes remaining = 194 (0x00C2)

ETHERNET: ETYPE = 0x0800 : Protocol = IP:  DOD Internet Protocol

ETHERNET: Destination address : 0009E84D0FA6

ETHERNET: .......0 = Individual address

ETHERNET: ......0. = Universally administered address

ETHERNET: Source address : 0009E84D07F2

ETHERNET: .......0 = No routing information present

ETHERNET: ......0. = Universally administered address

ETHERNET: Frame Length : 194 (0x00C2)

ETHERNET: Ethernet Type : 0x0800 (IP:  DOD Internet Protocol)

ETHERNET: Ethernet Data: Number of data bytes remaining = 180 (0x00B4)

IP: ID = 0x41; Proto = OSPF IGP; Len: 180

IP: Version = 4 (0x4)

IP: Header Length = 20 (0x14)

IP: Precedence = Internetwork Control

IP: Type of Service = Normal Service

IP: Total Length = 180 (0xB4)

IP: Identification = 65 (0x41)

IP: Flags Summary = 0 (0x0)

IP: .......0 = Last fragment in datagram

IP: ......0. = May fragment datagram if necessary

IP: Fragment Offset = 0 (0x0) bytes

page 35

Network Monitor trace  Thu 11/14/02 10:41:44  Capture1.txt

IP: Time to Live = 1 (0x1)

IP: Protocol = Open Shortest Path First IGP

IP: Checksum = 0x5FD3

IP: Source Address = 172.13.0.2

IP: Destination Address = 172.13.0.1

IP: Data: Number of data bytes remaining = 160 (0x00A0)

OSPF: Message = Link State Update

OSPF: Version = 2 (0x2)

OSPF: OSPF Packet Type = Link State Update

OSPF: Packet Length = 160 (0xA0)

OSPF: Source Router ID = 172.33.0.1

OSPF: Area ID = 0.0.0.1

OSPF: Checksum = 0x42D0

OSPF: Authentication Type = None

OSPF: Authentication = 0x0000000000000000

OSPF: Number of Ads = 4 (0x4)

OSPF: Age = 41 (0x29)

OSPF: Options = 34 (0x22)

OSPF: ......1. = Processes AS External routes

OSPF: .....0.. = Does not support MOSPF

OSPF: ....0... = Does not process Not-So-Stubby-Area advertisements

OSPF: ...0.... = Does not process external attributes LSAs

OSPF: ..1..... = Supports demand circuit extensions

OSPF: Link State Advertisement Type = Router Links

OSPF: Router ID = 172.33.0.1

OSPF: Router ID = 172.33.0.1

OSPF: Sequence Number = 2147483649 (0x80000001)

OSPF: Link State Checksum = 0x55AF

OSPF: Length = 48 (0x30)

OSPF: ..0..... = Not a virtual link endpoint

OSPF: .0...... = Not an AS Boundary Router

OSPF: 0....... = Not an area border Router

OSPF: Number of Links = 2 (0x2)

OSPF: IP network/Subnet number = 172.23.0.0

OSPF: Stub Network mask = 255.255.0.0

OSPF: Link Type = Connects to Stub Network

OSPF: Number of Metrics = 0 (0x0)

OSPF: TOS0 Metric = 1 (0x1)

OSPF: IP network/Subnet number = 172.13.0.0

OSPF: Stub Network mask = 255.255.0.0

OSPF: Link Type = Connects to Stub Network

OSPF: Number of Metrics = 0 (0x0)

OSPF: TOS0 Metric = 1 (0x1)

OSPF: Age = 21 (0x15)

OSPF: Options = 34 (0x22)

OSPF: ......1. = Processes AS External routes

OSPF: .....0.. = Does not support MOSPF

OSPF: ....0... = Does not process Not-So-Stubby-Area advertisements

OSPF: ...0.... = Does not process external attributes LSAs

OSPF: ..1..... = Supports demand circuit extensions

OSPF: Link State Advertisement Type = Summary Link (IP Network)

OSPF: Network Address = 172.43.0.0

OSPF: Router ID = 172.33.0.1

page 36

Network Monitor trace  Thu 11/14/02 10:41:44  Capture1.txt

OSPF: Sequence Number = 2147483649 (0x80000001)

OSPF: Link State Checksum = 0xB1E2

OSPF: Length = 28 (0x1C)

OSPF: Destination Netmask = 255.255.0.0

OSPF: SLA Type of Service = 1

OSPF: Metric = 2

OSPF: Age = 31 (0x1F)

OSPF: Options = 34 (0x22)

OSPF: ......1. = Processes AS External routes

OSPF: .....0.. = Does not support MOSPF

OSPF: ....0... = Does not process Not-So-Stubby-Area advertisements

OSPF: ...0.... = Does not process external attributes LSAs

OSPF: ..1..... = Supports demand circuit extensions

OSPF: Link State Advertisement Type = Summary Link (IP Network)

OSPF: Network Address = 172.33.0.0

OSPF: Router ID = 172.33.0.1

OSPF: Sequence Number = 2147483649 (0x80000001)

OSPF: Link State Checksum = 0x207F

OSPF: Length = 28 (0x1C)

OSPF: Destination Netmask = 255.255.0.0

OSPF: SLA Type of Service = 1

OSPF: Metric = 1

OSPF: Age = 21 (0x15)

OSPF: Options = 34 (0x22)

OSPF: ......1. = Processes AS External routes

OSPF: .....0.. = Does not support MOSPF

OSPF: ....0... = Does not process Not-So-Stubby-Area advertisements

OSPF: ...0.... = Does not process external attributes LSAs

OSPF: ..1..... = Supports demand circuit extensions

OSPF: Link State Advertisement Type = Summary Link (IP Network)

OSPF: Network Address = 172.3.0.0

OSPF: Router ID = 172.33.0.1

OSPF: Sequence Number = 2147483649 (0x80000001)

OSPF: Link State Checksum = 0x9D1E

OSPF: Length = 28 (0x1C)

OSPF: Destination Netmask = 255.255.0.0

OSPF: SLA Type of Service = 1

OSPF: Metric = 3

00000:  00 09 E8 4D 0F A6 00 09 E8 4D 07 F2 08 00 45 C0   ..èM.¦..èM.ò..EÀ

00010:  00 B4 00 41 00 00 01 59 5F D3 AC 0D 00 02 AC 0D   .´.A...Y_Ó¬...¬.

00020:  00 01 02 04 00 A0 AC 21 00 01 00 00 00 01 42 D0   ..... ¬!......BÐ

00030:  00 00 00 00 00 00 00 00 00 00 00 00 00 04 00 29   ...............)

00040:  22 01 AC 21 00 01 AC 21 00 01 80 00 00 01 55 AF   ".¬!..¬!..?...U¯

TRAMA RECONOCIMIENTO ACTUALIZACIÓN

Frame    Time         Src MAC Addr    Dst MAC Addr    Protocol    Description

334      288.858000   0009E84D07F2    USC IN000005    OSPF        Message = Link State

Src Other Addr     Dst Other Addr     Type Other Addr

172.13.0.2         224.0.0.5

Frame: Base frame properties

Frame: Time of capture = 14/11/02 10:27:48.612

Frame: Time delta from previous physical frame: 144000 microseconds

Frame: Frame number: 334

Frame: Total frame length: 78 bytes

Frame: Capture frame length: 78 bytes

Frame: Frame data: Number of data bytes remaining = 78 (0x004E)

ETHERNET: ETYPE = 0x0800 : Protocol = IP:  DOD Internet Protocol

ETHERNET: Destination address : 01005E000005

ETHERNET: .......1 = Group address

ETHERNET: ......0. = Universally administered address

ETHERNET: Source address : 0009E84D07F2

ETHERNET: .......0 = No routing information present

ETHERNET: ......0. = Universally administered address

ETHERNET: Frame Length : 78 (0x004E)

ETHERNET: Ethernet Type : 0x0800 (IP:  DOD Internet Protocol)

ETHERNET: Ethernet Data: Number of data bytes remaining = 64 (0x0040)

IP: ID = 0x49; Proto = OSPF IGP; Len: 64

IP: Version = 4 (0x4)

IP: Header Length = 20 (0x14)

IP: Precedence = Internetwork Control

IP: Type of Service = Normal Service

IP: Total Length = 64 (0x40)

page 53

Network Monitor trace  Thu 11/14/02 10:41:44  Capture1.txt

IP: Identification = 73 (0x49)

IP: Flags Summary = 0 (0x0)

IP: .......0 = Last fragment in datagram

IP: ......0. = May fragment datagram if necessary

IP: Fragment Offset = 0 (0x0) bytes

IP: Time to Live = 1 (0x1)

IP: Protocol = Open Shortest Path First IGP

IP: Checksum = 0x2C48

IP: Source Address = 172.13.0.2

IP: Destination Address = 224.0.0.5

IP: Data: Number of data bytes remaining = 44 (0x002C)

OSPF: Message = Link State Ack

OSPF: Version = 2 (0x2)

OSPF: OSPF Packet Type = Link State Ack

OSPF: Packet Length = 44 (0x2C)

OSPF: Source Router ID = 172.33.0.1

OSPF: Area ID = 0.0.0.1

OSPF: Checksum = 0xE442

OSPF: Authentication Type = None

OSPF: Authentication = 0x0000000000000000

OSPF: Age = 5 (0x5)

OSPF: Options = 34 (0x22)

OSPF: ......1. = Processes AS External routes

OSPF: .....0.. = Does not support MOSPF

OSPF: ....0... = Does not process Not-So-Stubby-Area advertisements

OSPF: ...0.... = Does not process external attributes LSAs

OSPF: ..1..... = Supports demand circuit extensions

OSPF: Link State Advertisement Type = Router Links

OSPF: Router ID = 172.43.0.2

OSPF: Router ID = 172.43.0.2

OSPF: Sequence Number = 2147483650 (0x80000002)

OSPF: Link State Checksum = 0x72D4

OSPF: Length = 48 (0x30)

00000:  01 00 5E 00 00 05 00 09 E8 4D 07 F2 08 00 45 C0   ..^.....èM.ò..EÀ

00010:  00 40 00 49 00 00 01 59 2C 48 AC 0D 00 02 E0 00   .@.I...Y,H¬...à.

00020:  00 05 02 05 00 2C AC 21 00 01 00 00 00 01 E4 42   .....,¬!......äB

00030:  00 00 00 00 00 00 00 00 00 00 00 05 22 01 AC 2B   ............".¬+

00040:  00 02 AC 2B 00 02 80 00 00 02 72 D4 00 30         ..¬+..?...rÔ.0